Memory map for Proolix (80x86 real mode)

00000 Vectors

00400 ROM BIOS data

00500 Kernel PSP

00600 Kernel Begin              (see KERNEL_BEGIN in boot.c)

07C00 =0:7C00 (=0070:7500) Boot sector

07E00                      Boot sector end

xxxxx Kernel end

xxxxx+1 Begin of dynamic memory allocate area

30500 (=3050:0000) /boot (boot manager) (see boots.asm, PSPSeg constant)

MemTop (f.e. 9FFFF) End   of dynamic memory allocate area

A0000 EGA (in graph modes)

B0000 MDA, Hercules 1st videopage

B8000 CGA, EGA, VGA 1st videopage (mode=3, symbol mode)

C8000 Additional ROM modules (2K blocks)

E0000 End of addtn ROM

E0000 Bg  AT ROM BIOS

EFFFF End AT ROM BIOS

F6000 ROM Basic

FE000 ROM BIOS, POST

FFFF0 JMP - COLD REBOOT

FFFF5 BIOS version/date (ASCII)

FFFFE PC/XT/AT identefication byte